ARDUINO CODE FOR NEXT GEN WATER MANAGEMENT USING IoT:

#define BLYNK_TEMPLATE_ID "TMPL3qSql3JLZ"

#define BLYNK_TEMPLATE_NAME "Pipe line system"

#define BLYNK_AUTH_TOKEN "MLkhdpEf9EzSXHYfBhWKtlI-RyAEIsD3"

#define BLYNK_PRINT Serial

#include <ESP8266WiFi.h>

#include <BlynkSimpleEsp8266.h>

char ssid[] = "user"; // your WiFi name

char pass[] = "12345678"; // your WiFi password

#define SENSOR_COUNT 4

#define LED_PIN 2

#define LEAK_VPIN V9

#define LEAK_LABEL_VPIN V10

struct Sensor {

const char* name;

uint8_t pin;

volatile unsigned long pulseCount;

float totalLitres;

float flowRate;

int vPinFlow;

int vPinTotal;

};

// Assign sensor pins and Blynk virtual pins

Sensor sensors[SENSOR_COUNT] = {

{"Ward 1", 4, 0, 0.0, 0.0, V1, V2},

{"Ward 2", 5, 0, 0.0, 0.0, V3, V4},

{"Ward 3", 12, 0, 0.0, 0.0, V5, V6},

{"Ward 4", 13, 0, 0.0, 0.0, V7, V8}

};

const float pulsesPerLiter = 450.0;

unsigned long lastMillis = 0;

const unsigned long intervalMs = 1000;

void IRAM_ATTR pulseISR0() { sensors[0].pulseCount++; }

void IRAM_ATTR pulseISR1() { sensors[1].pulseCount++; }

void IRAM_ATTR pulseISR2() { sensors[2].pulseCount++; }

void IRAM_ATTR pulseISR3() { sensors[3].pulseCount++; }

BlynkTimer timer;

void setup() {

Serial.begin(115200);

Serial.println("\nNodeMCU starting...");

for (int i = 0; i < SENSOR_COUNT; i++) pinMode(sensors[i].pin, INPUT_PULLUP);

attachInterrupt(digitalPinToInterrupt(sensors[0].pin), pulseISR0, FALLING);

attachInterrupt(digitalPinToInterrupt(sensors[1].pin), pulseISR1, FALLING);

attachInterrupt(digitalPinToInterrupt(sensors[2].pin), pulseISR2, FALLING);

attachInterrupt(digitalPinToInterrupt(sensors[3].pin), pulseISR3, FALLING);

pinMode(LED_PIN, OUTPUT);

digitalWrite(LED_PIN, HIGH); // LED OFF

WiFi.begin(ssid, pass);

Serial.print("Connecting to WiFi");

unsigned long start = millis();

while (WiFi.status() != WL_CONNECTED && millis() - start < 10000) {

delay(500); Serial.print(".");

}

if (WiFi.status() == WL_CONNECTED) {

Serial.println("\nWiFi connected!");

Serial.print("IP: "); Serial.println(WiFi.localIP());

Blynk.begin(BLYNK_AUTH_TOKEN, ssid, pass);

} else {

Serial.println("\nWiFi failed. Running offline.");

}

lastMillis = millis();

}

void loop() {

if (WiFi.status() == WL_CONNECTED) Blynk.run();

unsigned long now = millis();

unsigned long elapsed = now - lastMillis;

if (elapsed >= intervalMs) {

noInterrupts();

unsigned long pulseCopy[SENSOR_COUNT];

for (int i = 0; i < SENSOR_COUNT; i++) {

pulseCopy[i] = sensors[i].pulseCount;

sensors[i].pulseCount = 0;

}

interrupts();

float intervalSec = elapsed / 1000.0;

Serial.println("----- Water Flow Readings -----");

for (int i = 0; i < SENSOR_COUNT; i++) {

float freqHz = pulseCopy[i] / intervalSec;

float flowLmin = freqHz / 7.5;

if (flowLmin < 0.02) flowLmin = 0.0;

sensors[i].flowRate = flowLmin;

sensors[i].totalLitres += pulseCopy[i] / pulsesPerLiter;

Serial.print(sensors[i].name);

Serial.print(": Flow = ");

Serial.print(flowLmin, 3);

Serial.print(" L/min, Total = ");

Serial.print(sensors[i].totalLitres, 3);

Serial.println(" L");

if (WiFi.status() == WL_CONNECTED) {

Blynk.virtualWrite(sensors[i].vPinFlow, flowLmin);

Blynk.virtualWrite(sensors[i].vPinTotal, sensors[i].totalLitres);

}

}

Serial.println("-------------------------------");

// --------- Leak Detection (Imbalance method) ----------

float avgFlow = 0;

for (int i = 0; i < SENSOR_COUNT; i++) avgFlow += sensors[i].flowRate;

avgFlow /= SENSOR_COUNT;

bool leakDetected = false;

float threshold = 0.5; // ignore noise

float leakFactor = 1.5; // 50% higher than average

String leakMsg = "";

for (int i = 0; i < SENSOR_COUNT; i++) {

if (sensors[i].flowRate > avgFlow * leakFactor && sensors[i].flowRate > threshold) {

leakMsg += "🚨 Leak in " + String(sensors[i].name) + "\n";

leakDetected = true;

}

}

if (leakDetected) {

Serial.println(leakMsg);

digitalWrite(LED_PIN, LOW); // LED ON

if (WiFi.status() == WL_CONNECTED) {

Blynk.virtualWrite(LEAK_VPIN, 255);

Blynk.virtualWrite(LEAK_LABEL_VPIN, leakMsg);

Blynk.logEvent("leak_alert", leakMsg);

}

} else {

digitalWrite(LED_PIN, HIGH);

if (WiFi.status() == WL_CONNECTED) {

Blynk.virtualWrite(LEAK_VPIN, 0);

Blynk.virtualWrite(LEAK_LABEL_VPIN, "✅ No Leak Detected");

}

}

// -----------------------------------------------------

lastMillis = now;

}

}